# Standard Operating Procedures for

# Chest X-ray Reading

| SOP Number: SP-009-ET | Effective Date: 21- Jan 2019 |
| --- | --- |
| Version Number and Date: 1.1 , Date 19-Nov-2018 |  |

Table of contents

1.	PURPOSE	2

2.	SCOPE	2

3.	RESPONSIBLE FUNCTIONS	2

4.	DEFINITIONS and ABBREVIATIONS	2

5.	PROCEDURE	3

5.1	Material	3

5.2	Chest X-ray reading	3

5.3	Common chest X-ray findings in tuberculosis	5

6.	REFERENCES	8

7.	SUPPORTING DOCUMENTS	8

8.	APPENDIX	8

# Standard Operating Procedures for

# Chest X-ray Reading

## PURPOSE

This standard operating procedure (SOP) describes the procedure for chest x-ray reading. It presents the basic principles for adequate interpretation of chest X-ray and give examples of the most common intrathoracic lesions due to tuberculosis in adults.

## SCOPE

This SOP is developed for trained healthcare workers (clinicians and clinical investigators) taking care of patients in the endTB Observational study and in the endTB clinical trials.

## RESPONSIBLE FUNCTIONS

| Function | Activities |
| --- | --- |
| Medical doctor, clinical officer, site principal Investigator and/or site clinical investigator | - To read and interpret findings on the chest X-ray |

## DEFINITIONS and ABBREVIATIONS

The gold standard for the positioning of general chest radiography is ‘Erect’ meaning the patient is standing, and the X-ray beam direction is ‘Posterior-Anterior. Lateral view might give some additive information, especially on the localization of segmental lesions and the existence of lymphadenopathy.

Below is an example of a normal chest X-ray (Image 1).

Image 1:

Image 2:

Left

Right

Left lung is divided into 2 lobes and right lung into 3 lobes

3

5

6

1

4

2

but for the localisation of the lesions each lung is divided

into 3 zones as shown on Image 2: Upper (1&6), Middle (2&5)

and Low (3&4) zones.

## PROCEDURE

### Material

- X-ray viewing box (for analog X-rays)

- Computer with viewing software (for digital X-ray)

### Chest X-ray reading

For a good interpretation of the chest X-ray the following basic conditions must be met:

- Full-size antero-posterior chest radiographs must be taken.

- First assess the quality of the radiography considering 4 different aspects:

  - Identification marking of the patient. The date of the X-ray and the patient identification (name in routine program and Patient ID number for studies) should be clearly marked on the X-ray. For patients in the clinical trials, the name of the patient should be hidden using a sticker.

  - Patient positioning:

    - Check that there is no unpenetrated lung field

    - Check rotation by looking at the clavicle head ends and by measuring the rib ends projecting over the lung fields and comparing the two sides, which should be similar. In alternative, it is also possible to check the distance of the clavicle heads form the spinous process, which should be similar bilaterally.

    - Check that position of the clavicles of the patient is placed at any part of the 4th posterior rib bone on the X-ray film.

    - Check penetration: correct penetration is ensured when the intervertebral spaces can just be distinguished through the heart shadow.

    - Check inspiration: adequate inspiration is when the 8th-9th posterior rib, or the 6th anterior rib, is visible. If you find that all of the lower line of the 10th rib (usually of the right lung) is seen below the line of the right diaphragm, this means that there was a poor inspiration.

  - Density and Contrast

    - Three structures should be black:

      - the trachea and the bronchi: Follow the trachea and bronchi carefully, looking for displacement or narrowing

      - the right and left lung: pulmonary vessels should be easily traced to the lung periphery

      - the stomach bell: look to ensure that the gas shadow in the stomach does not extend into the chest (hernia).

    - Three structures should be white:

      - Soft tissue: Examine the soft tissue of the chest for swelling or lumps. The pulmonary vessels should be easily traced behind the cardiac shadow.

      - Bony structures: Examine the bony tissue for fractures, signs of rickets, or areas of infiltration.

      - Heart shadow: Examine the cardiac shadow for position, size, and shape.

  - Presence of external object or artefacts.

    - Artefacts are unwanted imaging mainly caused by mishandling of X-ray film, dirty stains on the intensifying screen, or improper processing of X-ray film. There should be no artefacts in the lung fields.

If the quality of the chest X-ray file is not adequate for interpretation, request a new X-ray. Chest X-ray with defective lung fields and poor inspiration should not be interpreted.

### Common chest X-ray findings in tuberculosis

#### Parenchymal lesions

Image 4:

Image 3:

Cavitation in upper left zone.

Bilateral heterogeneous consolidation of the upper zones

Image 5:

Image 6:

Infiltration of the upper right zone

Atelectasis of the right lung                                                          .

Nodular lesions

Image 8:

Image 7:

Tuberculoma in the left upper zone

Diffuse spread miliary pattern

#### Pleural abnormalities

Image 9:

Bilateral pleural effusion

#### Central structures

Image 10:

Image 11:

Mediastinal lymph node enlargement

Mediastinal and hilar lymph node enlargement

#### Other lesions

Image 13:

Image 12:

Destroyed right lung: TB sequels

Clacified pleura : TB sequels

Image 14:

Trapped left lung due to tuberculous empyema

## REFERENCES

- HANDBOOK for District Hospitals in Resource Constrained Settings for the Quality Improvement of Chest X-ray Reading in Tuberculosis Suspects. Tuberculosis Coalition for Technical Assistance (TBCTA), 2010.

http://www.tbcare1.org/publications/toolbox/tools/access/XRayQIHandbook.pdf

- HANDBOOK for District Hospitals in Resource Constrained Settings on Quality Assurance of Chest Radiography: for better TB control and health system strengthening. Tuberculosis Coalition for Technical Assistance (TBCTA), 2008.

http://www.tbcare1.org/publications/toolbox/tools/access/XRayQAHandbook.pdf

## SUPPORTING DOCUMENTS

None

## APPENDIX

None